|  | PRINCIPAL HULL DATA | ENCL. |  |
| --- | --- | --- | --- |
|  |  | REPORT |  |
|  |  | DATE | 2026-06-06 |
|  |  | REF |  |

SHIP:	GNC Supply1

Loading condition:	Design WL

Draught AP/FP:	5.956  / 6.044  [m]

Symbol	Unit	           	——————————————————————————————————————————————————————————————

Length overall	LOA	[m]	     88.015

Length on designed waterline	LWL	[m]	     83.093

Length betw. perp.	LPP	[m]	     82.800

Breadth moulded	B	[m]	     19.200

Breadth waterline	BWL	[m]	     19.200

Depth to 1st deck	D	[m]	     14.000

Draught at LPP/2	T	[m]	      6.000

Draught at FP	TFP	[m]	      6.044

Draught at AP	TAP	[m]	      5.956

Trim (pos. aft)	t	[m]	     -0.089

Rake of keel		[m]	      0.000

Rise of floor		[m]	      0.000

Bilge radius		[m]	      0.000

——————————————————————————————————————————————————————————————

Sea water density	s	[kg/m3]	    1025.00

Shell plating thickness		[mm]	          2

Shell plating in % of displ.		[%]	       0.40

——————————————————————————————————————————————————————————————

Volume displacement		[m3]	     6200.9

Displacement		[t]	     6381.3

Prismatic coefficient*	CP	[-]	     0.6545

Block coefficient*	CB	[-]	     0.6501

Midship section coefficient	CM	[-]	     0.9933

Longitudinal C.B. from LPP/2	LCB	[m]	     -5.261

Longitudinal C.B. from LPP/2*	LCB	[% LPP]	     -6.354

Longitudinal C.B. from AP	LCB	[m]	     36.139

Wetted surface	S	[m2]	    2011.15

Wetted surface of transom stern	AT	[m2]	      18.40

——————————————————————————————————————————————————————————————

Remarks:  *Refers to LPP

Hydrostatic corrections not included

ShipX (RepGen version 2.0.19) 08-Aug-2007 18:54:00 - Licensed to: NTNU

|  | HYDROSTATICS | ENCL. |  |
| --- | --- | --- | --- |
|  |  | REPORT |  |
|  |  | DATE | 2026-06-06 |
|  |  | REF |  |

SHIP:	GNC Supply1

Loading condition:	Design WL

Draught AP/FP:	5.956  / 6.044  [m]

Symbol	Unit	           	——————————————————————————————————————————————————————————————

Length overall	LOA	[m]	     88.015

Length betw. perp.	LPP	[m]	     82.800

Breadth moulded	B	[m]	     19.200

Depth to 1st deck	D	[m]	     14.000

Draught at LPP/2	T	[m]	      6.000

Draught at FP	TFP	[m]	      6.044

Draught at AP	TAP	[m]	      5.956

Trim (pos. aft)	t	[m]	     -0.089

Rake of keel		[m]	      0.000

Rise of floor		[m]	      0.000

Bilge radius		[m]	      0.000

——————————————————————————————————————————————————————————————

Sea water density	s	[kg/m3]	    1025.00

Shell plating thickness		[mm]	          2

Shell plating in % of displ.		[%]	       0.40

——————————————————————————————————————————————————————————————

Length on waterline	LWL	[m]	     83.093

Breadth waterline	BWL	[m]	     19.200

Volume displacement		[m3]	     6200.9

Displacement		[t]	     6381.3

Prismatic coefficient*	CP	[-]	     0.6545

Block coefficient*	CB	[-]	     0.6501

Midship section coefficient	CM	[-]	     0.9933

Longitudinal C.B. from LPP/2	LCB	[m]	     -5.261

Longitudinal C.B. from LPP/2*	LCB	[% LPP]	     -6.354

Longitudinal C.B. from AP	LCB	[m]	     36.139

Vertical C.B.	VCB	[m]	      3.326

Wetted surface	S	[m2]	    2011.15

Wetted surface of transom stern	AT	[m2]	      18.40

Waterplane area	AW	[m2]	    1389.50

Waterplane area coefficient	CW(LWL)	[-]	      0.871

Longitudinal C.F. from LPP/2	LCF	[m]	    -10.569

Longitudinal C.F. from AP	LCF	[m]	     30.831

——————————————————————————————————————————————————————————————

Immersion	DP1	[t/cm]	     14.242

Trim moment	MT1	[t·m/cm]	     89.065

Transverse metacenter above keel	KMT	[m]	      9.401

Longitudinal metacenter above keel	KML	[m]	    116.027

——————————————————————————————————————————————————————————————

Remarks:  *Refers to LPP

Hydrostatic corrections not included

ShipX (RepGen version 2.0.19) 08-Aug-2007 18:54:05 - Licensed to:  NTNU

|  | DATA-CHECK PROPERTIES | ENCL. |  |
| --- | --- | --- | --- |
|  |  | REPORT |  |
|  |  | DATE | 2026-06-06 |
|  |  | REF |  |

Run name:

Ship name:GNC Supply1

Loading condition description: Design waterline

ShipX exported data

Main dimensions (from input):

______________________________________________________________

Length between perpendiculars    (m)     82.800

Breadth                          (m)     19.200

Draught, midship                 (m)      6.000

Sinkage                          (m)      0.000

Trim, + = aft                  (deg)      0.000

______________________________________________________________

Hull data (from geometry):

______________________________________________________________

Hull sections                           4 -  40

Hull length                      (m)     82.800

Hull breadth at mid section      (m)     19.200

Hull draught at mid section      (m)      6.000

______________________________________________________________

Coefficients for data check etc.:

Type                                  Specified    Calculated

______________________________________________________________

Displacement                (tonnes)    6360.00       6362.21*

Vertical center of bouyancy,     KB                     3.334*

Vertical center of gravity,      VCG      7.300*

Longitudinal center of bouyancy, LCB                   36.014*

Longitudinal center of gravity,  LCG     36.000        36.014*

Block coefficient,               Cb       0.650         0.651

Water plane area coefficient,    Cw       0.871         0.870

Prismatic coefficient,           Cp                     0.656

Mid section area coefficient,    Cm       0.993         0.990*

Longitudinal metacentric height, GMl                  103.628*

Transverse metacentric height,   GMt                    2.144*

Roll radius of gyration,         r44      6.720*

Pitch radius of gyration,        r55     20.700*

Yaw radius of gyration,          r66     20.700*

Roll-yaw radius of gyration,     r46      0.000*

______________________________________________________________

* - Applied in the hydrodynamic calculations

ShipX - 2007-08-08 - 6:54:49 PM - Licensed to: NTNU

|  | HULL GRID | ENCL. |  |
| --- | --- | --- | --- |
|  |  | REPORT |  |
|  |  | DATE | 2026-06-06 |
|  |  | REF |  |

ShipX - 2007-08-08 - 6:54:49 PM - Licensed to: NTNU